Image Enhancement using Polymer Grid Triode Arrays

Alan J. Heeger, David J. Heeger*, John Langan and Yang Yang

UNIAX Corporation

6780 Cortona Rd.

Santa Barbara, CA 93117

Abstract

An array of polymer grid triodes (PGTs) connected through a common grid functions as a “plastic retina” which provides local contrast gain control for image enhancement.  This simple device, made from layers of conducting polymers, functions as an active resistive network that performs center-surround filtering. The PGT array with common grid is a continuous analog of the discrete approach of Mead, with a variety of fabrication advantages and with a significant saving of "real estate" within the unit cell of each pixel.

When a person views a brightly lighted external scene through a window from inside a poorly lighted room, the individual has no difficulty seeing, simultaneously, the details of both the internal scene and the external scene. This is done by local contrast control; the visual system locally adjujsts the gain using lateral inhibition.

Consider the office scene in Figure 1. Fig. 1a is the original (14 bit) image, but displayed using only the dynamic range available on the printed page (approximately 8 bits). Gray regions greater than 255 were clipped (set to 255), simulating saturation in the region of highest brightness. In Fig. 1b, the same image was rescaled (the intensity of each pixel was divided by 8) and displayed using the dynamic range available on the printed page (again, gray values greater than 255 were clipped). Fig. 1b is thus analogous to the image shown on a video display with reduced gain; the features are visible only in the background (bright) regions. In both cases, a great deal of information contained in the original image has been lost; in Fig. 1a, the viewer cannot see any detail in the bright regions of the image, in Fig. 1b the viewer cannot see any detail in the darker regions of the image.

* Permanent address: Department of Psychology, Stanford University, Stanford, CA 94305

Local contrast control involves a combination of logarithmic compression and lateral inhibition, the latter provided by a horizontal resistive network (a “neural network”) (1, 2). After logarithmic compression, the output from a given pixel (Vi) is proportional to the log of the intensity (Ii) of that pixel:

Vi = VologIi	(1)

The lateral inhibition is implemented by subtracting from Vi the average of the surrounding values; thus, the renormalized image is defined by

vi = VologIi - <VologIi>ave = VologIi - (Vo/N)Σi logI	(2)

where the sum is over the neighboring pixels within the averaging range (the center surround or blurring range). Eq 2 is equivalent to:

(3)

The denominator is the geometric mean. Eq 3 is the mathematical expression which defines Mead’s local contrast enhancement algorithm (1); the equation generalizes in the obvious way, to allow for a weighted average by replacing 1/N by wi where wi are the weights.

The effect of processing the the original image of the office scene with the Mead algorithm is shown in Fig. 1c.  Figure 1c was obtained by renormalizing the 14 bit image with Eq 3 using a Gaussian weighted average over an 13x13 neighborhood. In Fig. 1c, one can see details in the entire image even within the limited range available on the printed page. The simulations in Fig. 1 demonstrate the power of local contrast enhancement.

In this report, we show that the local contrast enhancement algorithm (Eqs. 2 and 3) can be implemented with a simple device made from layers of conducting polymers. The device consists of an array of polymer grid triodes (3, 4) connected through a common grid which serves as a resistive network.

The layered thin film PGT is shown schematically in Fig. 2a; the top layer (5) is the anode, the bottom layer (1) is the cathode.  The third electrode (3), analogous to the grid in a vacuum tube triode, is an open network of polyaniline (PANI) protonated to the highly conducting form

with camphor sulfonic acid (CSA) (3, 4). Semiconducting polymer forms the two layers, (2) and (4), between the anode and the polymer grid and between the polymer grid and the cathode, and it fills the void spaces within the porous PANI-CSA network (3’).

An array of PGTs with a common grid (Fig. 2b) can perform local contrast enhancement like that simulated in Fig. 1.  Since the thin films which constitute the PGT array can be processed from solution, they can be layered directly on an array of photodetectors. Each node of the PGT array corresponds to one pixel of the image. The array of PGTs with common grid performs three important functions:

(i) The common grid functions as a resistive network that computes the blurring/averaging in Eq. 2.

(ii) The output current at one node of the PGT array is  approximately the difference between the input anode-to-cathode voltage and the local grid voltage (see inset of Fig. 4).  Since the local grid voltage is the local average (i), the PGT array acts as a center-surround filter (1, 2) that computes the difference in Eq. 2.

(iii) The common grid PGT array provides the high input resistance needed for open circuit detector operation (5), which results in the logarithmic compression in Eq. 2.

Thus, local contrast enhancement, as described by Eqs. 2 and 3, can be directly implemented with the PGT array shown in Fig. 2b. The effective computation rates involved are impressive.  The equivalent calculation implemented on a serial computer would require about 40 million mulitplications per second (256x256 pixels, 30 frames per second, 13x13 blurring using separable convolution).

Polymer grid triode arrays, with four triodes on a single substrate all with a common grid, have been fabricated. The structure of this linear array is shown schematically in Fig. 2c. The fabrication process for the triode array is similar to that of a single PGT (3, 4); the principal difference is that there are separate contact pads for each device in the array.  For the arrays fabricated in this initial study, the sheet resistance of the common grid was approximately 20 KΩ/square.

The voltage of the common grid (VG) with respect to the anode was measured while a voltage (VAC) was applied between the anode and the cathode on the left triode (see Fig. 2c). The voltage of the common-grid was measured near the neighboring triode on the far right; VG at this neighboring position responds in proportion to VAC, showing that VG = β<VAC>av; that is, the grid voltage responds to the local input and provides a local average.

The final step required to demonstrate the validity of the polymer grid triode implementation of local contrast enhancement is to show that it computes the center-surround difference in Eq. 2.  The equivalent circuit of the PGT demonstrated previously (3, 4) is that of two coupled diodes connected back-to-back, like that of a bipolar transistor (5).  This is achieved by using semiconducting polymer in layers (2), (3’) and (4).  For the prototype array sketched in Fig. 2c,  layer (2) was fabricated with a material with sufficient conductivity to make an ohmic contact to the grid so that the equivalent circuit is simplified to a diode in series with a resistor.  In the initial experiments, polyvinylcarbazole (PVK) was used for this resistor layer (2).  In forward bias,

I = Ioexp[γ(VAC - VG)] + (VAC - VG)/Ri + VG/Rs	(4)

where γ is a constant (3, 4), Ri is the internal series resistance of the diode (due to the bulk resistivity of the semiconducting material used in (4)) and Rs is the series resistance resulting from the PVK layer, (2). Since the semiconducting layer is fabricated from a high resistivity, pure semiconducting polymer such as poly(phenylene vinylene), PPV, or one of its soluble deriviatives, Rs << Ri. Thus, the output of the PGT is a function of (VAC - VG) only; that is,

I = F(VAC - VG).	(5)

Figure 4 demonstrates that this is in fact the case: I vs VAC data are shown for different VG. As expected, the I vs VAC curves are sensitive to VG; for example, at VAC ≈ 5 V, the current can be suppressed from 1 mA to zero by changing VG. In the inset to Fig. 4, we have replotted the curves from the two limiting data sets (VG = -11 V and VG = +13.2 V) as a function of (VAC - VG). Since the forward bias data collapse onto a single curve, Eq 5 is indeed valid.

Consider then the array of PGTs, with common grid, sketched in Fig. 2b, and assume, for simplicity, that the common grid is grounded at infinity. By utilizing the common grid,

VG = β<VAC>ave	(6)

where <VAC>ave denotes the average over neighboring pixels with a distance determined by the sheet resistance of the grid and the conductance to ground of the resistive layer (in this case, PVK). The characteristic length over which the average is taken is given by (1)

L = 1/[ρσ]1/2	(7)

where ρ is the sheet resistance of the PANI network grid and σ is the conductance per unit area to ground through the resistive PVK layer.

We conclude that the current output from each pixel of the array (Ii) is given by

Ii = F [VAC(i) - β<VAC>ave] ≈ [VAC(i) - β<VAC>ave][∂F/∂V]	(8)

Eq 8 is equivalent to Eq 3 provided the output of each detector on the focal plane array, which serves as input to an individual pixel, is proportional to the logarithm of the intensity; VA(i) ∝ logIL(i) where IL(i) is the intensity of the light  incident on the ith pixel. Since Vout ∝ logIL for photovoltaic detectors under open circiuit conditions, the logarithmic compression of Eq 1 is straightforward (5).

The polymer grid triode array image processor differs fundamentally from those built with discrete silicon field-effect transistors (1, 2). The PGT array makes use of the spreading resistance of the PANI control grid network to provide the interconnection of a given node to its neighbors; the conductivity of the PANI network enables center surround filtering as a result of lateral charge redistribution initiated by contrast differences. Charge redistribution through a continuous layer of material provides a natural means for averaging/blurring.

By controlling the concentration of polyaniline in the network, one can control the resistivity over many orders of magnitude (6). Similarly, by varying the thickness and the resistivity of layer (2), by back biasing the grid-to-ground diode during operation, or by making (2) a bilayer which functions as a diode using conducting polymers (7-  9), one can vary the conductance of layer (2) over a wide range. The latter is particularly interesting since it allows dynamic control,

in situ, of the spatial decay length. Thus, one can vary both ρ and σ in Eq 7 so as to be able to achieve values for L (either statically or dynamically) ranging from a few microns to 1 cm.

We have shown that this simple device, made from layers of conducting polymers, provides both logarithmic compression and lateral inhibition of response, as required for local contrast control. Nevertheless, the plastic retina is at an early stage of development. The utility of the PGT array for image enhancement will depend on a number of factors, including for example, sensitivity, noise, dynamic range, and matching from one pixel to the next, etc. that must be tested on an engineering prototype.

To build a full plastic retina, the polymer grid triode image enhancement array would be fabricated directly onto the output (back) side of a photodetector array (for example an infrared detector array) using each detector output pad as the anode or cathode of the PGT at that node. The semiconductor layers would be cast sequentially from solution and applied onto the detector array much like an anti-reflection coating. The final contrast enhanced output would be connected to a demultiplexer by “bump-bonding”; that is by cold-welding indium bumps arrayed reciprocally on the PGT array output and on the demultiplexer input.

Alternatively, the PGT array could be utilized to process the image after analog to digital conversion and integrated directly into a display (such as an LCD display). In this case, the PGT array would be fabricated directly on, as an integral part of, the display; for example, between the control circuits and the liquid crystal layer. The data would be logarithmically compressed digitally and in-put into the PGT array to process the image; the output from the pixels of the array of polymer grid triodes would serve as the input to the pixels of the display.

References:

1. C. Mead, Analog VLSI and Neural Systems, (Addison Wesley, 1989).

2. Vision Chips: Implementing Vision Algorithms with Analog VLSI Circuits, Ed. by C. Koch and H. Li, IEEE Computer Science Press, 1994.

3. Y. Yang and A.J. Heeger, US patent application, Serial No. 08/227,979.

4. Y. Yang and A.J. Heeger, Nature, 372, 244(1994).

5. S.M. Sze, Physics of Semiconductor Devices, Wiley, New York, 1981.

6.  M. Reghu, C.O. Yoon, C.Y. Yang, D. Moses, P. Smith, A.J. Heeger, and Y. Cao, Phys. Rev. B50, 13931 (1994).

7. I.D. Parker, J. Appl. Phys. 75, 1656 (1994).

8. See for example, G. Yu, K. Pakbaz and A.J. Heeger, Appl. Phys. Lett. 64, 3422 (1994); and references therein.

9. H. Tomozawa, D. Braun, S.D. Phillips, and A.J. Heeger. Synth. Met. 22, 63, (1987).

10. The office scene image in Figure 1 was generated using the Radiance computer graphics rendering program, developed by Greg Ward, funded by the Lighting Group at Lawrence Berkeley Laboratory, the U.S. Department of Energy, and the Laboratory d’Energie Solaire et de Physique du Batiment (LESO-PB) at the Ecole Polytechnique Federale de Lausanne (EPFL) in Switzerland.

Figure 1:

a. Original (14 bit) image of an office scene displayed using only the dynamic range available on the printed page (approx. 8 bits). Gray values greater than 255 were clipped (set to 255). Features are visible only in the foreground (dark) regions.

b. The same image, rescaled (the intensity of each pixel was divided by 8), and again displayed using the dynamic range available on the printed page (again, gray values greater than 255 were clipped). Features are clearly visible only in the (bright) background.

c. The result of renormalizing the office scence image with local contrast gain control using Eq 3. Features are now visible throughout the image.

(a)   	(b)

(c)

Figure 2:

a. Structure of the polymer grid triode with the various layers. (1) and (5) are the cathode and anode (pixel) arrays, respectively.  The other layers are continuous films common to all  the PGTs within the array: (2) and(4) are semiconducting layers, poly(2-methoxy-5-(2'-ethyl-hexyloxy)-1,4-phenylene vinylene), MEH-PPV, and (3) is the common grid network filled with semiconductor (3').

b.  Schematic diagram of an array of PGTs with a common grid.  Thickness (between anode and cathode) is approximately 0.3 µm.  Each anode/cathode pad is, for example, 50 µm on a side.

c.  Four polymer grid triodes in an array with a common grid.

Figure 3: VAC is voltage applied between the anode and cathode on the left of  the array (see Fig. 2c).  VG is grid voltage measured near rightmost triode of the array (see Fig. 3c).  Grid voltage is proportional to input voltage applied at neighboring positions, i.e., the grid voltage provides a local average.

Figure 4: I vs VAC for different grid voltages (VG). The inset shows the two limiting data sets (VG = -11 V and VG = +13.2 V) as a function of (VAC - VG).